MPB SERIES CP1200 FEATURES

- VERY LOW CALIBRATION SCALES TO 2cc

- WIDE RANGE OF STANDARD CAPACITIES FROM 2cc to 20cc

(SEE FULL RANGE OVERLEAF)

- EASY TO READ

- CALIBRATED SCALES WITH TRACEABLE CERTIFICATES

- SPECIAL SCALES ‘DOWN READ’ TO CUSTOMERS

SPECIFICATION IF REQUIRED

- THREADED, OR FLANGED CONNECTIONS

- PRECISION BORE BOROSILICATE GLASS CYLINDERS

- POLYCARBONATE COVERS AS STANDARD

- PERMANENTLY FIRED SCALES WITH WHITE

BACKING TO GIVE EXCELLENT CLARITY

- PUMP SETTING GAUGE

- PUMP RATE CHECKING

- CALIBRATION TUBE EASILY REMOVED IN SITU FOR CLEANING

- FOR CAPACITIES FROM 50ml to 250ml SEE MPBTB 031 (CP100)

- FOR CAPACITIES FROM 500cc to 10,000ml SEE MPBTB 077 (CP110)

GENERAL INFORMATION

This range of precision calibration cylinders have been designed to meet the requirements of high accuracy and robust construction where metering low capacity outputs is essential.

This product range is usually supplied with threaded connections; however, flanges can be

supplied if specified.

Ideal for use as a low flow pump setting gauge, or pump rate checking.

Materials of construction include 316 Stainless steel, super duplex Stainless steel, titanium, Hastelloy, rigid PVC and polypropylene. These are typical of the specifications MPB can provide.

MPB SERIES CP1200 CALIBRATION CYLINDER

Product Data Sheet

100mm

100mm

100mm

100mm

100mm

100mm

100mm

MPVA 807

MPVA 807

MPVA 807

MPVA 807

MPVA 807

MPVA 807

MPVA 807

DRAWING NUMBER

End Connections:

316 Stainless steel, Super Duplex

Stainless steel, Titanium, Hastelloy,

Rigid PVC and Polypropylene

Threaded, or flanged to customer

requirements.

Fully certified where requested.

Seals:	    Viton, Nitrile, EPDM, etc

Cylinder:          Fully stress relieved precision bore

Borosilicate glass

Scale:	      Permanently fired black ceramic

Maximum  Working Pressure: 13.3 Barg

Maximum  Working Temperature:

60°C RPVC

100°C Stainless steel

Accuracy:   2% of actual reading

SCALE LENGTH

¼” NPT

¼” NPT

¼” NPT

¼” NPT

¼” NPT

¼” NPT

¼” NPT

20ml

15ml

10ml

7.5ml

5ml

3ml

2ml

CAPACITY

Alternative capacities and connections are available,

please contact MPB Sales Office

with precise requirements

CONNECTION

MPB Industries Ltd - Designers and Manufacturers of Scientific and Process Control Instrumentation

# Part of the Scientific Digital Imaging plc (SDI) group of companies

# Unit 1 Branbridges Ind Est, East Peckham, Kent TN12 5HF, UK, Tel: +44 (0)1622 872401

# mail@mpbflowmeters.com, www.mpbflowmeters.com

For additional information on the Sight Level Indicator or any of our flowmeter products,

please call our Sales Office on (01622) 872401.

MPB SERIES CP1200 CALIBRATION CYLINDER

Due to the constant development and improvement of products, information may be altered or withdrawn without notice.